PRODUCT SPECIFICATION

GPS RECEIVER BOX

# Specification

Product Feature

- SiRFstar I high performance and low power  consumption signal processor

- All-in-view 12-channel parallel processing

- Built-in active antenna

- High sensitivity to satellite signal

- Cold start under 45 seconds, average

- Superior urban canyon performance

- FoliageLock for weak signal tracking

- Build-in SuperCap to reserve system data for rapid satellite acquisition.

- USB interface connection port

System Specification

- Electrical Characteristics

| <Receiver> |  |
| --- | --- |
| - Frequency | L1, 1575.42MHz |
| - C/A code | 1.023MHz chip rate |
| - Channels | 12 |
| - Sensitivity | -170dBW |
| <Accuracy> |  |
| - Position Horizontal | 25m 95% (SA off), |
| - Velocity | 0.1m/sec 95% (SA off), |
| - Time | 1microsecond synchronized to GPS time |
| <Datum> | WGS-84 |
| <Acquisition Rate> |  |
| - Reacqusition | 0.1 sec., average (recovery time for being interrupted) |
| - Hot start | 8 sec., average (with ephemeris and almanac valid) |
| - Warm start | 38 sec., average (with almanac but not ephemeris) |
| - Cold start | 45 sec., average (neither almanac nor ephemeris) |
| <Dynamic Condition> |  |
| - Altitude | 18,000 meters (60,000 feet) max. |
| - Velocity | 515 meters/sec. (1000knots) max. |
| <Power> |  |
| - Voltage supply | 5V±5% |
| - Current supply | 180mA typical |
| - GPS Protocal | SiRF binary/NMEA 0183 changeable(Default:NMEA) |
| - GPS Output Data | SiRF binary >> position, velocity, altitude, status and control NMEA 0183 (v2.2) >> GGA, GSA, GSV, RMC(VTG and GLL are optional) |
| - GPS transfer rate | Software command setting (Default : 4800,n,8,1 for NMEA and 19200,n,8,1 for SiRF binary ) |

# Electrical Characteristics

| <Temperature> |  |
| --- | --- |
| - Operating | -15 deg. C to +70 deg.C |
| - Available | - 40 deg. C to +80 deg.C |
| - Storage | - 40 deg. C to + 80 deg.C |

| USB Port GPS | RS232 Serial Port GPS |
| --- | --- |

Due to continuous product improvements, all specifications may be subject to change without notice