PC Corporation

Software Department

Software Release Notes

| Title of Program | Pen Driver for ST3400 (Windows 2000) |
| --- | --- |
| Version | 1.4.0.0 |
| Release Date | 05-14-2001 |
| Author | Uday A. Prabhune |

Confidential

This document contains confidential information of technical and commercial nature and should not be disclosed to any person or party without expressed written permission of Fujitsu PC Corp.

PROGRAM INFORMATION

| Platform(s) | BIOS (or N/A) | Operating System(s) |
| --- | --- | --- |
| Stylistic 3400 | 1.04 or later | Windows 2000 |

| Filename(s) | Size(s) in Bytes | Modified Date(s) | Time(s) |
| --- | --- | --- | --- |
| FUJPEN.SYS | 20,208 | May 14, 2001 | 3:07:13 PM |
| PENCAL.DLL | 90,112 | December 12, 2000 | 2:48:34 PM |
| PENCFG.CPL | 61,440 | September 14, 2000 | 2:43:10 PM |
| PENMON.EXE | 49,152 | December 13, 2000 | 11:36:01 AM |
| FUJPEN.CAT | 0 | June 29, 2000 | 11:36:00 AM |
| FUJPEN.INF | 7,226 | May 14, 2001 | 3:30:37 PM |
| PENPREP.EXE | 221,184 | December 14, 2000 | 1:05:12 PM |

Description of Program (and associated files)

The above files are required to enable the pen, in Windows 2000 on the ST3400. Following is the description of the driver files : FUJPEN.SYS Windows 2000 Pen Device Driver. PENCAL.DLL Dynamic Link Library that communicates with the calibration control panel applet and the pen device driver to provide pen calibration. PENCFG.CPL Pen calibration control panel applet. PENMON.EXE Program that runs in the background and upon receiving the WM_DISPLAYCHANGE message (change in display resolution or display rotation) communicates with the device driver to provide the new settings. FUJPEN.CAT WHQL digital signature for the driver package. Note: The size of this file is currently 0bytes because only after the qualifications, the driver package will be sent to WHQL for certification. FUJPEN.INF Installation file for the device driver. #### PENPREP.EXE Pre-installation (preparation) program. This program backs-up the original driver files and then copies new updated files to the default installation folder (C:\FPC\Install\PenDrv).

Installation and/or Operating Instructions

Installation involves: - Executing the PenPrep.Exe (pre-installation) program, - Updating the pen driver using control panel mouse applet and - Reinstalling the PenX driver (if desired for inking and recognition) to work with the new driver. Note: To update the pen driver, you need to be logged in as administrator (Administrator Mode). In User Mode, you will not have the privileges to update the device drivers. Step A: (Installation Preparation): ### Execute the PenPrep.Exe program. This program backs-up the original driver files and then copies new updated files to the default installation folder (C:\FPC\Install\PenDrv). That way, the original files are backed in back-up folder (C:\FPC\Install\PenDrv\Bak), if need be. ### If PenX is installed, disable the PenX interaction with Pen Driver ### 1.Open the PenX Handwriter Setting applet in the control panel. 2.Disable the Recognition and Inking option under the Recognition tab. ### Step B (Pen Driver Update): #### Update the Pen Driver 1. Open the Mouse applet in the control panel. 2. Select the Hardware tab in the applet. 3. In the Devices list, select the CIC PenX Filter Driver… (if PenX is installed) or Fujitsu Stylistic 3400 Pen device (if PenX is not installed). 4. Click the Properties button. The driver properties dialog will be displayed. 5. Select the Driver tab. 6. Click the Update Driver button. This launches the Upgrade Device Driver Wizard. 7. Click the Next botton. 8. Select Display a list of the known drivers … 9. Click the Next button. This displays Select a Device Driver dialog. 10. Click on the Have Disk button. This displays the Install from Disk dialog. 11. Click on Browse button. This displays Locate File dialog. 12. Select the Fujpen.Inf file in the “C:\FPC\Install\PenDrv” folder. Note that the file extension may not be displayed in the locate dialog. 13. Click Open button. This gets you back to Install from Disk dialog with “C:\FPC\Install\PenDrv” folder selected for Copy manufacturer’s files from option. 14. Click OK button. This gets you back to Select a Device Driver dialog. 15. Click Next button. Start Device Driver Installation dialog is displayed. 16. Click Next button. If the device driver is not WHQL certified at the time installation is being performed, Digital Signature Not Found dialog is displayed. Click Yes button to proceed with installation. This dialog will not be displayed once the driver is WHQL signed. 17. The driver will be updated and Completing the Upgrade Device Driver Wizard dialog will be displayed. 18. Click the Finish button. This takes you back to the Driver Properties dialog. You should see the Driver Version updated to reflect the new version. 19. Click the Close button. You will be prompted to restart your computer. Remove the driver diskette from the drive and then click Yes to restart the computer. Note that in case you are not prompted to restart computer, it is recommended that you do restart your computer. 20. When the computer restarts, you may see a CIC PenX dialog with Tablet not initialized… message. This is because the PenX driver is not yet installed to work with the updated pen driver. Click OK. #### Step C (PenX Driver Install): #### Install the PenX filter driver 1. Open the Mouse applet in the control panel. 2. Select the Hardware tab in the applet. 3. In the Devices list, select the Fujitsu Stylistic 3400 Pen device. 4. Click the Properties button. The driver properties dialog will be displayed. 5. Select the Driver tab. 6. Click the Update Driver button. This launches the Upgrade Device Driver Wizard. 7. Click the Next botton. 8. Select Display a list of the known drivers … 9. Click the Next button. This displays Select a Device Driver dialog. 10. Click on the Have Disk button. This displays the Install from Disk dialog. 11. Click on Browse button. This displays Locate File dialog. 12. Select the Moufiltr.Inf file in the “C:\Program Files\CIC” folder. Note that the file extension may not be displayed in the locate dialog. 13. Click Open button. This gets you back to Install from Disk dialog with “C:\Program Files\CIC” folder selected for Copy manufacturer’s files from option. 14. Click OK button. This gets you back to Select a Device Driver dialog. 15. Click Next button. Update Driver Warning dialog is displayed with Installing this device driver is not recommended… message. Click Yes to proceed. 16. Start Device Driver Installation dialog is displayed. 17. Click Next button. Digital Signature Not Found dialog is displayed. Click Yes button to proceed with installation. This dialog will not be displayed once the CIC driver is WHQL signed. 18. Files Needed dialog may be displayed. From the pull down list for Copy files from option, select the “C:\FPC\INSTALL\PENDRV”. 19. Click OK button. The driver will be updated and Completing the Upgrade Device Driver Wizard dialog will be displayed. 20. Click the Finish button. This takes you back to the Driver Properties dialog. 21. Click the Close button. You will be prompted to restart your computer. Click Yes to restart the computer. Note that in case you are not prompted to restart computer, it is recommended that you do restart your computer. 22. When the computer restarts, there should be no warning messages related to pen and the pen and PenX should be working.

Command Line Options/Menu List (if applicable)

N/A

Examples (if applicable)

N/A

Testing Guidelines:

Make sure that the pen is calibrated at log-on screen after tablet is booted in display-rotated mode.

Known Deficiencies (if applicable)

MODIFICATION HISTORY

| Date | Version | Description of Modification(s) |
| --- | --- | --- |
| 09/14/2000 | 1.2.0.0 | Author: Uday Arun Prabhune Added calibration support and support of different resolutions SVGA/XGA screen display resolutions supported: 640 x 480 800 x 600 1024 x 768 1280 x 1024 |
| 12/14/2000 | 1.3.0.0 | Author: Uday Arun Prabhune 1. Added pen support in rotated display modes. Pen and pen calibration should now work at any of the resolutions and rotation angles listed below on both SVGA and XGA screen types. Rotation Angles 0 (display not rotated) 90 180 270 Display Resolutions 640 x 480 800 x 600 1024 x 768 1280 x 1024 (At this resolution, pen works – that is aligns with user input tap, and functions-, but panning not supported) 2. Added calibration support in rotated modes. |
| 05/14/2001 | 1.4.0.0 | The Trident display driver prior to 9525DVD 5.6072-03.26r did not retain rotation across reboots (for example if you rotate in 270degrees rotated mode, the tablet may boot back in 90degrees rotated mode etc). The pen driver therefore always assumed that the tablet is going to boot in landscape (0 degrees rotation) mode. After the tablet boots-up, pen supported rotation. Now that the Trident display driver 9525DVD 5.6072-03.26r is available, it seems to be retaining the rotation across reboots. So, this release of pen driver supports pen calibration across reboots – that way the pen is calibrated when the tablet boots-up in rotated mode. |